Isolated Full Brıdge DC/DC Converter

# Abstract

The aim of the work is desinging 96W full-bridge isolating converter with Vin= 12V and Vout=48V with 1% output voltage ripple. Switching frequency of the converter is 100kHz.

Contents

Isolated Full Brıdge DC/DC Converter	1

1	Abstract	1

2	Circuit Parameters	3

3	Magnetic Design	10

3.1	Design your transformer with cores which can be found on Magnetics. Clearly indicate the reasoning behind choosing the core. Find the primary and secondary number of turns. Calculate the magnetizing inductance of the transformer. (Hint: Magnetics has some great design guides for inductor and transformer designs. Please have a look at those guidelines.)	11

3.1.1	Number of turns calculations:	15

3.1.2	Magnetizing Inductance Calculations:	16

3.1.3	With current density being 4 A/mm2, find an AWG cable according to the switching frequency and current value. Firstly, calculate the fill factor of the transformer with the chosen cable. Is it a reasonable value? Then, calculate the cable DC and AC resistances and copper loss of the transformer (Proximity can be ignored).	16

3.1.4	Fill factor calculation:	18

3.1.5	Copper loss calculation:	19

3.1.6	Calculate the core loss of the transformer using core loss formulas given by the manufacturer and compare it with previously found copper loss value. Does your design require new iterations?	20

3.2	Design the output inductor with cores on Magnetics. Find the required number of turns and make sure that core is not saturated by checking the DC bias curves of the material.	20

3.2.1	Compute the product of LI² where	21

3.2.2	Number of turns calculation:	22

3.2.3	Choose appropriate AWG cable for the design with 4 A/mm2 current density and calculate the fill factor. Is it a reasonable value? Then, calculate the cable DC and AC resistances and copper loss of the inductor.	23

3.2.4	Copper loss calculation of inductor:	23

3.2.5	Calculate the core loss of the inductor and compare it with previously found copper loss value. Does your design require new iterations?	25

4	SIMULATION	26

5	References	29

# Circuit Parameters

- Assuming operating duty cycle is 0.4, find the turns ratio of the transformer.

- Find the required inductance value so that the filter inductor current ripple is 10% of the average inductor current.

- Find the output capacitor value to meet the output voltage ripple requirements. Do not just use the formula, show derivaton.

−

+

−

+

−

vx

+

SW4

SW3

SW2

SW1

Ns

Np

vp

D2

D1

R

C

Lx

Vo

Vd

Figure 1. Isolated Full-Bridge Converter

SW1 &  SW2 are operating in simultaneously. Besides, SW3 & SW4 are operating in simultaneously.

When SW1 & SW2 are ON and SW3 & SW4 are OFF:

vx

+

−

Lx

C

R

Vo

Vd

SW1

SW2

SW3

SW4

Np

Ns

D1

D2

+

−

Vd

Vo

+

−

C

R

Lx

+

vx

−

D1

D2

SW1

SW2

SW3

SW4

vp

+

−

Np

Ns

vp

+

−

Figure 2. Isolated Full-Bridge SW1 & SW2 ON, SW3 & SW4 OFF

When SW1 & SW2 are OFF and SW3 & SW4 are ON:

Figure 3. Figure 2. Isolated Full-Bridge SW1 & SW2 OFF, SW3 & SW4 ON

As it can be observed from the figures above, inductor voltage is positive in the both cases, therefore; the third case should be added and it should let discharge the inductor.

D1

Lx

−

+

VS

vx

−

−

+

+

Vo

R

C

D2

Figure 4. Isolated Full-Bridge SW1,  SW2, SW3 & SW4 OFF

vp

0

Here, voltage induced on secondary side cancels each other and vLx is being discharged.

vLx=VS-Vo

vLx=-Vo

t

DTs

Ts

DTs

Ts2

Ts2+D

D

Ts

Ripple frequency of inductor current is twice of the main frequency.

Vd

-Vd

t

vx

NsNpVd

As the ripple frequency is twice of main signal.

Vo=2VdNsNpD

The inductor must discharge its energy to not charging up continuously and damage to circuit eventually. There are 2 charge period in the inductor like it can be seen on the graph above. Therefore, duty cycle can’t be larger than D=0.5. However, there is no discharging period/time; so, D must be smaller than half of the period (D<0.5) to discharge properly.

As it is given:

Vin= 12V

Vout=48V

P=96W

fs=100kHz

- Assuming operating duty cycle is 0.4, find the turns ratio of the transformer.

Vo=2VdNsNpD

48=2×12×NsNp×0.4

NsNp=489.6=5

- Find the required inductance value so that the filter inductor current ripple is 10% of the average inductor current.

To get required inductance value, we need to have voltage waveform of the inductor:

vLx

t

NsNpVd-Vo

-Vo

D

Ts2

Ts2+D

Ts

vLx=LdiLxdt=L∆iLx∆T

L=vLx×∆T∆iLx

From the negative portion of the inductor voltage:

vLx=-Vo and ∆T=(0.5-D)Ts

Average inductor current is

ILx=Io= PoVo=96W48V=2A

∆iLx=0.1×2A = 0.2

L=Vo×(0.5-D)Ts0.2

L=Vo×(0.5-D)0.2×fs where fs=100kHz, and D=0.4

L=48×(0.5-0.4)0.2×100k=240μF

- Find the output capacitor value to meet the output voltage ripple requirements. Do not just use the formula, show derivaton.

ic

To get required capacitance value, we need to have current waveform of the capacitor:

t

iLx-Io

Io-iLx

D

Ts

iLx

It is not easy to calculate capacitance value from this graph. Therefore, it is possible to derive capacitance value from the technique is used in buck converter.

t

Io,avg

D

Ts2

∆iLx

Ts4

∆Q

Electrical capacitance says Q=CV

∆Q=C∆V

∆Q=∆iLx2×Ts42=∆iLx×Ts16

∆V=∆QC

∆V=∆iLx×Ts16C

∆iLx=VLx×∆TLx

Using off-time ∆iLx equation:

∆iLx=Vo×(0.5-D)TsLx

∆V=Vo×(0.5-D)Ts×Ts16LxC

∆Vo=Vo×(0.5-D)Ts216LxC=Vo×(0.5-D)Ts216LxC=Vo×(0.5-D)16LxCfs2

∆VoVo=(0.5-D)16LxCfs2 or 1-2D32LxCfs2

It is given that we have 1% ripple at the output voltage.

0.01=1-2D32LxCfs2   ,where D=0.4, Lx=240μF, fs=100kHz

C=260nF

# Magnetic Design

The total loss of magnetic components -transformer and inductor- should not exceed 15 W and fill factor of the cores should not be less than 30%.

It is strongly advised to use programs such as Matlab or Excel, since magnetic design can be a tedious job which may require a few iterations to find the optimum parameters. A simple script would save you from losing your precious hours.

a)  Design your transformer with cores which can be found on Magnetics. Clearly indicate the reasoning behind choosing the core. Find the primary and secondary number of turns. Calculate the magnetizing inductance of the transformer. (Hint: Magnetics has some great design guides for inductor and transformer designs. Please have a look at those guidelines.)

b) With current density being 4 A/mm2, find an AWG cable according to the switching frequency and current value. Firstly, calculate the fill factor of the transformer with the chosen cable. Is it a reasonable value? Then, calculate the cable DC and AC resistances and copper loss of the transformer (Proximity can be ignored).

c) Calculate the core loss of the transformer using core loss formulas given by the manufacturer and compare it with previously found copper loss value. Does your design require new iterations?

d) Design the output inductor with cores on Magnetics. Find the required number of turns and make sure that core is not saturated by checking the DC bias curves of the material.

e) Choose appropriate AWG cable for the design with 4 A/mm2 current density and calculate the fill factor. Is it a reasonable value? Then, calculate the cable DC and AC resistances and copper loss of the inductor.

f) Calculate the core loss of the inductor and compare it with previously found copper loss value. Does your design require new iterations?

## Design your transformer with cores which can be found on Magnetics. Clearly indicate the reasoning behind choosing the core. Find the primary and secondary number of turns. Calculate the magnetizing inductance of the transformer. (Hint: Magnetics has some great design guides for inductor and transformer designs. Please have a look at those guidelines.)

First, let’s try to understand basics:

Faraday’s law:

For each turn of wire:

vturn(t)=dΦdt

Total winding voltage is

v(t)=NdΦdt

v(t)

+

-

i(t)

core

Φ(t)

Core area Ac

Express flux Φ in terms of flux density Φ=BacAc

Vpri=NpriAcdBac(t)dt

Vpri=NpriAcBac2πf2π

VpriIpri=4NpriAcBacfIpri

Ipri is not one of the specificationsof transformer, therefore we can write instead:

I=JAw           where Aw is the area of copper (wire)

VpriIpri=4NpriAcBacfJrmsAw

Fill factor Ku

Copper area Aw

Window area WA

We are using constant 2 because having 2 winding in the transformer

However, Aw is still not related to the transformer specifications, therefore, we can write instead:

Ku=2NAwWA

Aw=KuWA2N

To simplify the equation, following equation can be used which is taken from Magnetics.Inc:

WAAc=PoDcmaKtBmaxf

Where Kt = Topology constant

Forward converter = 0.0005

Push-Pull = 0.001

Half-bridge = 0.0014

Full-bridge = 0.0014

Flyback = 0.00033 (single winding)

Flyback = 0.00025 (multiple winding)

Dcma=1J

Dcma= Current density (cir. mils/amp)

Current density can be selected depending upon the amount of heat rise allowed. 750 cir. mils/amp is conservative; 500 cir. mils is aggressive.

Insert Aw into the equation:

VpriIpri=4NpriAcBacfJrmsAw

VpriIpri=4NpriAcBacfJrmsKuWA2Npri

VpriIpri=2fKuBacJrmsAcWA

AcWA=VpriIpri2KuJrmsBacf

WAAc=PoDcmaKtBmaxf

Our specifications:

Po = 96W

Kt for full-bridge = 0.0014

f=100kHz

Figure 5. Flux density (gauss) vs frequency (kHz)

Suggested interval for Dcma is between 500 to 750 cir. mils/amp

Dcma=750 cir. mils/amp

Bmax= Flux Density (gauss) selected based upon frequency of operation. Above 20kHz, core losses increase. To operate ferrite cores at higher frequencies, it is necessary to operate the core flux levels lower than ± 2 kg. The Flux Density vs. Frequency chart shows the reduction in flux levels required to maintain 100 mW/cm³ core losses at various frequencies, with a maximum temperature rise of 25°C. for a typical power material, Magnetics’ P material. From transformer design with magnetics ferrite cores (Mag. Inc)

Figure 6. Flux density (gauss) vs frequency (kHz)

From the graph above, it is suitable for using the value Bmax=1000 (gauss)

Tesla (T) = 10,000 Gauss

WAAc=PoDcmaKtBmaxf

WAAc=96×7500.0014×1000×100000=0.52 cm4

Now, it is time to choose suitable core for the design of transformer,

It is important that our core is able to handle power of 96W and meet WAAc = 0.52 cm4.

When we check Area Product Distribution (WAAc) Chart (Mag. Inc) and Typical Power Handle Chart (Mag. Inc)

WAAccm4

Figure 7. WaAc chart

| 20 kHz | 50 kHz | 100 kHz | 250 kHz |
| --- | --- | --- | --- |

Suitable cores are 43019 DS HS, 42515 EE, 43009 EE.

Figure 8. Typical power handle chart

We are working on 100kHz. We need at least 96W power ratings for our system. Hence, it is better to choose the bottom row.

Suitable cores are 42823 PC,43019 RS DS PC, 43009 EE.

43009 EE meets the requirements for WAAc  and handles the requirement power rating.

Now let’s check 43009 EE core in detail. It is a E core with part number 3009.

Figure 9. Magnetics Inc. material list

As it is decided that system will be working on 100mT (1,000 Gauss) and 100kHz.

According to Magnetics Inc. Ferrite Core Material Summary

Figure 10. Ferrite core material types

The cores have types like L, R, P, F, T

Power loss of material types can be observed on figure above. To make losses minimize, P or T types are better to use.

The choose is: P type 3009 E core.

### Number of turns calculations:

To simplify the equation, following equation can be used which is taken from Magnetics.Inc:

Np=Vp4BAcf108

Again from Faraday’s law of induction:

v(t)=NdΦdt

Vpri=NpriAcdBac(t)dt

Vpri=NpriAcBac2πf2π

Vpri=4NpriAcBacf

Npri=Vpri4AcBacf

Vp=12V

B=1000 Gauss (100 mT)

f=100 kHz

Effective cross-sectional area Ae=83.2 mm2

Np=124×1000×83.2×100000108=3.6 ≅ 4 turns

NsNp=5, hence Ns=20 turns

AL = inductance factor 1R

Magnetizing Inductance Calculations:

L=N2R=ALN2

N=LR=LAL

L=ALN2

AL value of 0P43009EC is 3147

Lm=50.4μH

### With current density being 4 A/mm2, find an AWG cable according to the switching frequency and current value. Firstly, calculate the fill factor of the transformer with the chosen cable. Is it a reasonable value? Then, calculate the cable DC and AC resistances and copper loss of the transformer (Proximity can be ignored).

Cable Selection:

Isec,rms=DIo2+2DIo22=1.55 A

Ipri,rms=12 A

Current density J is specified as 4A/mm2

Therefore;

Apri=12A/4A/mm2=3mm2

Asec=1.55A/4A/mm2=0.4mm2

Let’s check American Wire Gauge(AWG) table:

Figure 11. American Wire Gauge (AWG) table

Apri=12A/4A/mm2=3mm2

Asec=1.55A/4A/mm2=0.4mm2

In normal conditions, 12AWG for primary, 21AWG for secondary is suitable. But, we have a constraint of skin depth δ. The system switching frequency is 100kHz. As it can be seen on the above list, for both 12AWG and 21AWG, skin depth δ reach maximum value below 100kHz. Therefore, we need to use strands of small gauge which has skin depth δ is above 100kHz. For this purpose, 26AWG cable with having cross-sectional area 0.128mm2 is good solution. To get same cross-sectional area, for example, 24×26AWG in parallel for primary winding, 4×26AWG in parallel for secondary windings is necessary.

### Fill factor calculation:

Ku=Total copper areaWindow area

Ku=Aw,primary+2Aw,secondaryWA

Ku=3×(4 turns)+2×0.4×(20 turns)83.2=0.34

### Copper loss calculation:

Figure 11. 0P43009EC core dimensions

Np=4 turns

Ns=20 turns

Number of parallel cables for pri = 24

Number of parallel cables for sec = 4

(MLT) = 2πE-F2=2π21.41-9.42=37.73 mm

### Rpri=Npri×(MLT)×Rone cable(number of parallel cables)

### Rsec=Nsec×(MLT)×Rone cable(number of parallel cables)

Rpri=4×37.73×10-3×133.85×10-324=0.84mΩ

### Rsec=20×37.73×10-3×133.85×10-34=25mΩ

Pcu,total=Ipri,rms2Rpri+2×Isec,rms2Rsec

Pcu,total=122×0.84mΩ+2×22×25mΩ

Pcu,total=0.32W

### Calculate the core loss of the transformer using core loss formulas given by the manufacturer and compare it with previously found copper loss value. Does your design require new iterations?

To calculate core loss, we are using Steinmetz’ equation:

Pcore=αfxByVcore

The parameters depend on the material which is supplied by the manufacturer.

Vcore=5.150mm3

B=0.1T = 1000 Gauss

Typically,  1<x<3 and 2 <y<3

Pcore=2×3.53×1000001.42×0.12.88×5.1=0.6W

Ptotal=Pcu,total+Pcore=0.32+0.6 = 0.92W

Our system design is not required further iterations. Losses are kind of valid.

## Design the output inductor with cores on Magnetics. Find the required number of turns and make sure that core is not saturated by checking the DC bias curves of the material.

To design an inductor, two parameters must be known:

- Inductance required with DC bias

- DC current

Inductance was found as 240μH and DC average current of the inductor is equal to the output current Io=2A.

The procedure is:

### Compute the product of LI² where

- L = inductance required with DC bias (millihenries)

- I = maximum DC output current + 1/2 AC Ripple

Figure 12. Magnetics Inc. Ferrite core selection chart

The ripple is specified as 10%. Therefore, we take I as 2+%10×22 =2.05A

LI2=240×10-6×2.052=1 mJ

From above picture which shows ferrite core selection chart according to LI2 value.

We found LI2= 1mJ. Put the value into the chart and follow the coordinate up to intersection point. AL(mH/1000 turns) = 300 and core type is G (42213) which can be found on Magnetics Inc. Ferrite Pot Cores with part number 2213

Required inductance L, core type and core nominal inductance AL are known.

### Number of turns calculation:

N=103LAL

Where L is in millihenries.

N=103240×10-3300=28.28 ≅29 turns

Let’s recheck

L=N2AL        ( T2×nH/T2)

L=28.282×300×10-9=240μH

- Design the output inductor with cores on Magnetics. Find the required number of turns and make sure that core is not saturated by checking the DC bias curves of the material.

One of suitable core is Ferrite Pot Core part number 2213 with material type of P, having

AL=315

le (path length) = 31.5mm

Ae (cross-sectional area)=63.4mm2

To check saturation:

The maximum allowable H = 25 Oersted

le (path length) in cm = 3.15cm

NI (maximum)=0.8×H×le

NI (maximum) =0.8×25×3.15cm=63 A•T

So, it is in safe region.

### Choose appropriate AWG cable for the design with 4 A/mm2 current density and calculate the fill factor. Is it a reasonable value? Then, calculate the cable DC and AC resistances and copper loss of the inductor.

We have 2.2A passing through the inductor.

Ainductor=2.2A/4A/mm2=0.54mm2

From AWG list in page 16, 19AWG with 0.653mm2 is suitable; however, skin depth factor is occurs because our system is working on 100kHz.

Therefore, we can use parallel strands 26AWG with 0.128mm2 by the number of 5. Therefore, core losses can be minimized.

### Copper loss calculation of inductor:

### Rinductor=Ninductor×(MLT)×Rone cable(number of parallel cables)

Rone cable=133.8568×10-3Ω for 26AWG

Ninductor=29 turns

Figure 13. POT core dimensions

(MLT)=2πE-G2=2π17.9-3.82=44.3mm

### Rinductor=29×44.3×10-3×133.8568×10-35=34.4mΩ

Copper loss of inductor

Pcu=IL2Rinductor

Pcu=2.22×34.4mΩ=0.17W

### Calculate the core loss of the inductor and compare it with previously found copper loss value. Does your design require new iterations?

Core loss calculation of inductor:

Pcore(mW)=αfxByVe

Ve=2000mm3

f=100kHz

Typically,  1<x<3 and 2 <y<3

Pcore=3.53×1000001.42×0.12.9×2=0.1W

Ptotal=Pcore+Pcu=0.17+0.1=0.27W

All values seems to be valid.

# SIMULATION

Figure 14. Simulink schematic design of Full-Bridge DC-DC Converter

Figure 15. Transformer parameters

Figure 16. Inductor voltage waveform

Figure 17. Inductor current waveform

Figure 18. Output parameters

# References

1  (n.d.). Magnetic Design Guides. Magnetics. https://www.mag-inc.com/Design/Design-Guides