Non-Isolated dc/dc Converters

The aim of the work is to design a converter which reduces 16V DC to 12V DC. The design options that we have for the task are Buck-Boost, Ćuk and SEPIC converter. The converter should be able to supply 24W at switching frequency of 50kHz. The output voltage ripple should not exceed 2%. In the end, which one is best solution for the application will be determined.

Buck-Boost Converter

a) Analytically determine the value of inductance for converter to be in CCM operation with 10% ripple current.

b) Analytically find the output capacitance value in order to have 2% output voltage ripple.

c) According to your calculations, find commercial products for inductors, capacitors and semiconductors from Digikey. State the parameters of devices. Explain your reasoning for selections.

d) Validate your results by constructing the designed buck-boost converter in Matlab/Simulink environment with non-idealities, meaning ESR values of capacitors and inductors, which you can find in the datasheets of the products. You can use ideal switching devices. Plot the following waveforms. Comment on the results and explain if you observe any discrepancy from analytical calculations.

- Output voltage

- Input current

- Inductance current

io

−

+

−

+

iL

>

>

VL

Vo

S

Figure 1. Buck-Boost Converter

>

ic

+

−

−

+

iL

>

VL

Vo

Figure 2. Buck-Boost Converter when S is ON

>

io

+

−

+

−

iL

>

VL

Vo

Figure 3. Buck-Boost Converter when S is OFF

a.

OFF

ON

t

t

iL

B

A

(1-D).Ts

A

D.Ts

-Vo

Vd

VL

∆iL

Io,avg

b.

ton

Figure a: Inductor voltage waveform, Figure b: Inductor current waveform

In properly designed converters, charging and decharging in the inductor current should be equal. Otherwise, the inductor current increases all the time and may damage to circuit.

Therefore, Area A and Area B should be equal each other and sum of these will cancel each other.

A+B=0

Vd.D.Ts+(-Vo)(1-D)Ts=0

Vd.D-Vo+Vo.D=0

VoVd=D1-D                                   or                       Vo=D1-D Vd

Put the values into the equation.

Vd= 16V

Vo = 12V

VoVd=1216=D1-D             →           D=43%

- Analytically determine the value of inductance for converter to be in CCM operation with 10% ripple current.

vL=Ldidt→ vL=L∆i∆t

vL=Vd during D.Ts period

∆i=Vd.D.TsL or Vd×DL×fs

Ts can be written as Ts=1fs  , and fs is given as 50kHz.

∆iL=0.1=1L×Vd×Dfs

L=10.1×16×0.4350k→L=1.38mH

- Analytically find the output capacitance value in order to have 2% output voltage ripple.

ic

iL-Io

iL,max

iL,min

Q2

D.Ts

Ts

t

Q1

-Io

ic=CdVdt→ic=CΔVcΔT

For first period (Q1) → ic=Io

Io=CΔVcD.Ts

C=Io.D.TsΔVc

The output voltage ripple is given as 2%.

Therefore ΔVc = 0.02, Io = 24W12V=2A

ΔVc=Io.D.TsC → C= Io.D ΔVc.fs → C =2×0.450.02×50k → C=0.9mF

- According to your calculations, find commercial products for inductors, capacitors and semiconductors from Digikey. State the parameters of devices. Explain your reasoning for selections.

Capacitor and Inductor values were found in the previous pages. They are:

L=1.38mH

C=0.9mF

However, 1.38mH is not available for purchase. When the specifications are applied to search motor in Digikey, the possible inductor value is 2.2mH when the inductor current ratings are taken into account. Therefore, our inductor value will be 2.2mH.

The appropriate inductor is: CTDR4F-222K

It has DC resistance (DCR) 0.494Ω

IsatA=4.00A

Cost: Does not specified. Assumed it is $3/unit

For the capacitor, the required specifications are entered in Digikey, it will show us available capacitors. One of them is: 16SEPF1000M (Polymer Aliminum Solid Capacitor)

It has capacitance of 1mF, voltage rated of 16V, ESR of 12mΩ

Cost: ($2.32/unit)

For switching component, I preferred MOSFET. Which needs to have 30V voltage rate, and 4A current rate, which is available in Digikey

DMG3418L-7

Which has Rds(on) 60mΩ

Its price: $0.43

Diode has to be withstand 38V reverse. Average current flow through diode is 3.8A

Therefore, I chose 40V, 4A Schottky Diode for this purpose.

SSB43L-E3/52T

It has forward voltage (VF= 0.45V)

Its price is $0.6

- Validate your results by constructing the designed buck-boost converter in Matlab/Simulink environment with non-idealities, meaning ESR values of capacitors and inductors, which you can find in the datasheets of the products. You can use ideal switching devices. Plot the following waveforms. Comment on the results and explain if you observe any discrepancy from analytical calculations.

Figure 4. Buck-Boost Conerter with non idealities

Figure 5. Buck-Boost converter ideal case

Here in both cases, duty ratio (D) are calculated as 0.45 or 45%. However, with non-idealities, with the same duty ratio (D), it could not reach desired output voltage values. Therefore, duty ratio (D) has to be increased.

With increase in duty ratio (D) +7%. That is, 52% duty ratio provides the following waveform:

Figure 6. Increase in duty ratio (D) = 52%

Figure 7. Input Current with non-idealities

Figure 8. Input current in ideal case

As there are some power losses due to non-idealities, the duty ratio increased to ensure to get the same parameters. Therefore, with the increase in duty ratio (D), input current will be much higher than before. As it can be seen on the graphs above.

Figure 9. Inductor current in ideal case

Figure 10. Inductor current with non-idealities

As the input current is increased with increase in duty ratio (D), the inductor current has also increased. However, as inductance is different and bigger than in ideal case, the current ripple in the inductor is decreased as expected.

There are some differences due to non-idealities like ESR of capacitor, inductor, or voltage drops in switching components. Also, on-resistances of the switching components.

Ćuk Converter Design

C1

L2

L1

-

-

-

-

+

+

vc1

+

+

Q

R

C2

D

Vo

Vd

vL1

vL2

Figure 11. Ćuk converter circuit schematic

There are 2 modes in Ćuk converter, one is switch (Q) is ON and another one is switch (Q) is OFF.

OFF-State:

iL1

iL2

L2

C1

L1

-

-

-

-

+

+

+

+

Vo

vL2

Q

D

R

C2

vL1

vc1

Vd

Figure 12. Switch (Q) is OFF

When the switch (Q) is OFF, iL1 & iL2 flow through the diode. The capacitor C1 is charging through the diode by the energy from both Vd & L1. Current iL1 decreases since vc1> Vd. Energy stored in L2 feeds the output. Therefore, iL2 also decreases.

iL2

L1

C1

L2

ON-State:

vc1=Vd+Vo

vc1=Vd-(-Vo)

iL1

-

-

-

-

+

+

+

+

Vo

vL2

vc1

vL1

Q

D

Vd

C2

R

Figure 13. Switch (Q) is ON

As vc1> Vo, C1 is discharging through the switch, transferring energy to the output & L2. Also, reverse biases the diode. Therefore, iL2 increases. The input source Vd feeds L1 causing iL1 to increase.

The inductor current iL1& iL2 are assumed to be continuous. The voltage & current expressions is steady-state can be obtained in two different ways.

OFF-State

vL1=Vd-vc1 , (vc1>Vd)     →iL1 decreases

vL2=-Vo                                    →iL2 decreases

ON-State

vL1=Vd                  →iL1 increases

vL2=vc1-Vo       →iL2 increases

Vd

vL1

Ts

DTs

t

Vd-vc1

iL1

t

vL2

vc1-Vo

t

-Vo

iL2

t

From L1 volt-second balance equation:

VdDTs+Vd-vc11-DTs=0

VdDTs+VdTs-VdDTs-vc1Ts+vc1DTs=0

VdTs-vc1Ts+vc1DTs=0

VdTs=vc1Ts-vc1DTs
Vd=vc11-D

vc1=Vd1-D                         equation 1

From L2 volt-second balance equation:

vc1-VoDTs+-Vo1-DTs=0

vc1D-VoD-Vo+VoD=0

vc1=VoD                                equation 2

From equation 1 & 2

vc1=Vd1-D          &        vc1=VoD

VoD=Vd1-D

Transfer function of Ćuk converter is

VoVd=D1-D

IoId=1-DD

Where

IL1=Id

IL2=Io

Vo=VdD1-D

Ćuk converter can be operated in both buck converter or boost converter depends on the duty cycle (D). However, output voltage is a negative. It is a inverting converter.

To recap, in the buck converter, discontinuous input current we have. In the boost converter, we have discontinuous output current, in the buck-boost converter we have discontinuous current on both the input as well as on the output side.

That means, we encounter such EMI problems in discontinuous converters. However, with the Ćuk converter, EMI problems can be minimized. If EMI problems are important for a design, Ćuk converter is a better solution for it.

- Calculate the required output capacitor, C2 , value in order to have 2% voltage ripple, and find the values of L1, L2 inductances by assuming 10% ripple current.

C1

L1

C2

L2

Figure 14. Ćuk converter circuit schematic

It can be said that right side of the Ćuk converter is similar to buck converter as seen in above figure. Therefore, the voltage ripple at the output can be derived from buck converter:

ΔVo = Ts2.Vo1-D8L2C2

ΔVo=0.02=Vo1-D8L2C2fs2

C2=Vo1-D8L2×0.02fs2

To find the value of C2, L2 should be known.

We assume we are in CCM

Q2

Q1

Ts

DTs

∆iL1=VdDTsL1=vC1-Vd1-DTsL1

Q2 period

Q1 period

∆iL1=VdDTsL1=VdDL1fs=16×0.43L1×50k

Ripple at the inductor currents was given as 10%.

L1=16×0.43∆iL1×50k=16×0.430.1×50k

L1=1.38mH

Ts

DTs

Q2

Q1

∆iL2=vC1-VoDTsL2=Vo1-DTsL2

Q2 period

Q1 period

∆iL2=0.1=Vo1-DTsL2

∆iL2=0.1=12×1-0.43L2×fs

∆iL2=0.1=12×1-0.43L2×50k

L2=12×1-0.43∆iL2×50k

L2=12×1-0.430.1×50k

L2=1.37mH

!

The fact is ripple at the inductor currents are same, so inductor values should be same.

Let’s assure that the converter is working at CCM.

Id=IL1,avg

IL1,avg=D1-DIo,avg

Io,avg=PoVo=24W12V=2A

IL1,avg=0.431-0.43×2=1.51A

For both ON and OFF-States

IL2,avg=Io,avg=2A

∆iL1≪IL1,avg

∆iL2≪IL2,avg

Therefore, CCM is verified.

Percentage error of the inductors:

∆iL1IL1,avg=0.11.51=0.067=6.7%

∆iL2IL2,avg=0.12=0.05=5%

For capacitor C2

C2=Vo1-D8L2×0.02fs2

C2=121-0.438×1.38m×0.02fs2=12.4μF

C2=12.4μF

- Find the value of C1 capacitor by assuming 10% ripple voltage.

ic1=C1Δvc1ΔT, and ic1=Io during D period, ic1=IL1=Id during 1-D period.

Δvc1=IL1×1-DTsC1=Io×D×TsC1

IL1is found as 1.51A, Io is found as 2A, D = 0.43.

Δvc1=IL1×1-DC1fs=1.51×1-0.43C1×50k

C1=1.51×1-0.43Δvc1×50k=1.51×1-0.430.1×50k

C1=0.173mF≈0.18mF

To ensure the value is correct, let’s calculate from D period.

Δvc1=Io×D×TsC1=Io×DC1×fs

C1=Io×DΔvc1×fs=2×0.430.1×50k=0.173mF≈0.18mF

C1=0.18mF

As it supposed to be that C1≫C2

- According to your calculations, find commercial products for capacitors, inductors and semiconductors from Digikey. State the parameters of devices. Explain your reasoning for selections.

L1=L2=1.38mH

C2=12.4μF

C1=0.18mF

The inductor values are same with the buck-boost converter, so the inductor found in the buck-boost design can be used for Ćuk converter.

The appropriate inductor is: CTDR4F-222K

It has DC resistance (DCR) 0.494Ω

IsatA=4.00A

Cost: Does not specified. Assumed it is $3/unit

For 
C2=12.4μF

There is no available capacitance such 12.4μF. Therefore, the closest one is 20μF

TE1090-E3

Rated Voltage = 6V, C = 20μF

No ESR specified

Price: $4.84/unit

For

C1=0.18mF = 180μF

C1 should be able to handle Vd+Vo which is 16V + 12V = 28V

In Digikey, 180 μF 28V capacitor is not available; however, 180 μF 35V is available for purchase.

EEU-FR1V181B

ESR: 0.056Ω

Price: $0.68/unit

For MOSFET (Switch)

DMG3418L-7

VDSS=30V

ID=4 or 3.1A

ESR = 70mΩ

Price: $0.43/unit

For Diode

SK44BL-TP

VRMAX=40V

Ioavg=4A

VF=0.450V

Price: $0.45/unit

- Construct the Ćuk converter in Simulink with non-idealities and check if the circuit meets the requirements. Plot the following waveforms. Comment on the results and explain if you observe any discrepancy from analytical calculations.

- Output voltage

- C1 voltage

- L1,L2 currents

Ideal Case:

Figure 15. Output voltage waveform

Figure 16. Output voltage ripple in ideal case

Figure 17. C1 voltage & L1, L2 currents waveforms in ideal case

Figure 18. C1 voltage, L1 & L2 currents ripples in ideal case

With Non-idealities

Figure 19. Output voltage & current waveform with non-idealities

Figure 20. Output voltage ripple with non-idealities

Figure 21. C1 voltage, L1 & L2 current waveforms with non-idealities

Figure 22. Voltage and current ripples on C1, L1 & L2 with non-idealities

Results:

The results are close to the desired specifications. However, losses in the components due to ESR or any other non-idealities, It is expected to reduce voltage drops across the load or components. However, it can be observed that the ripples will remain same because there is no change in capacitance or inductance values. However, duty ratio is increased to 48%. (Was 45% in ideal case). Also, we can say that the time to reach steady-state condition is reduced.

SEPIC (Single Ended Primay Inductor Converter)

iL1

L1

−

+

iD

vC1

Q

−

−

L2

−

−

+

+

+

+

Io

iC2

iC1

iL2

Vo

Vd

D

R

vC2

C2

C1

vL2

vL1

Figure 23. SEPIC (Single Ended Primary Inductor Converter) circuit schematic

iL1

L1

−

+

vC1

−

+

+

−

L2

−

−

+

+

Io

iC2

iC1

iL2

Vo

Vd

R

vC2

C2

C1

vL2

vL1

Figure 24. SEPIC ON-state circuit

iL1

L1

−

+

iD

vC1

−

+

vL1

−

−

L2

−

+

+

+

iL2

Io

iC2

iC1

Vo

Vd

D

R

vC2

C2

C1

vL2

⦁

⦁

Figure 25. SEPIC OFF-state circuit

OFF-state:

VL1=-Vo

VL2=-Vo

To recap the equations:

ON-state:

VL1=Vd

VL2=Vd

During ON-state

VL1=Vd

VL2=VC1

From KVL:

-Vd+VL1+VC1+VL2=0

Volt-second balance must remains:

VL1=0        &      VL2=0

Therefore;

-Vd+VC1=0

VC1=Vd

Rewrite VL2 equation
VL2=Vd

During OFF-state

VL1=-Vd+VC1-Vo

VL2=-Vo

Rewrite VL1 equation:
VL1=-Vd+VC1-Vo

VL1=-Vd+Vd-Vo

VL1=-Vo

vL1

Ts

DTs

-Vo

-Vo

Vd

Vd

t

A

B

vL2

t

Volt-second balance equation must be continued to remain safety of the circuit.

Therefore, area A & area B must be equal in magnitude and cancel each other.

VdDTs+(-Vo)(1-D)Ts=0

VoVd=D1-D

It is same with Buck-Boost and Ćuk converter, but it is positive polarity at the output. In Buck-Boost and Ćuk, the converters have negative polarity at the output. Also, the difference here in SEPIC converter is ability to shutdown completely (When the switch is OFF).

iL1& iL2

Let’s see voltage & current waveforms to see the behaviour of the converter:

Ts

DTs

iC1

t

iL1

t

- iL2

iD

t

iL1+ iL2

iL1+ iL2

t

iQ

VQ

Vd+Vo

VD

Q is OFF

Q is ON

t

t

-Vo

For VQ, If Kirchhoff's Voltage Law is applied on the circuit below with yellow path:

+

−

VQ

Figure 26. SEPIC is in OFF-state

-VQ+VC1+VO=0

As we found VC1=Vd :

VQ=Vd+VO during OFF-state.

- Calculate the required output capacitor, C2 , value in order to have 2% voltage ripple, and find the value of L1, L2 inductances by assuming 10% ripple current.

To calculate the inductors L1 & L2:

First, it is necessary to find duty ratio (D) first.

As it is found in the previous pages:

VoVd=D1-D

1216=D1-D

D = 0.43 or 43%

VL1=L1diL1dt

VL1=L1ΔiL1ΔT

During ON-state (DTs):

VL1=Vd    & ΔT=DTs

L1=VdDTsΔiL1, where Ts=1fs

L1=16×0.430.1×50k=1.38mH

L1=L2=1.38mH

iC2

To calculate C2 values, we need to determine the current of the capacitor C2.

ID-Io

Q is OFF

Q is ON

-Io

t

ic2=C2dVcdt=C2∆Vc∆T

VC2=Vo

ic2=-Io during ON-state

∆t=DTs

C2=-IoDTs∆Vc

Io=24W/12V=2A

C2=2×0.430.02×50k

C2=860μF

- Find the value of C1 capacitor by assuming 10% ripple voltage.

To find C1, capacitor C1 current must be known.

iC1

iL1

-iL2

Ts

DTs

t

Figure 27. SEPIC circuit schematic

KCL at the yellow point:

-iC1-iL2+iD=0

iL2=iD-iC1

iD=iC2+Io

Substitute iD equation into iL2 equation:

iL2=iC2+Io-iC1

It is known that steady-state operation the capacitor net charge or charge-second balance on a capacitor is equal to zero.

Therefore; average current through the capacitor is iC2 & iC1 = 0

iL2=Io

Now, it is possible to calculate the capacitor C1

iC1=C1dVcdt=C1∆Vc∆T

C1=iC1×∆T∆Vc

Because average value of iL2 is found, the way of calculation became easier.

iC1=iL2=Io

Also, it is known that ripple at capacitor C1 is 10%.

C1=Io×DTs∆Vc,  Io can be replaced as Io=VoR and Ts=1fs

C1=VoDRfs∆Vc

C1=2×0.430.1×50k=172μF

C1=172μF

For switching components, Q needs to block positive voltage and positive current, D should block negative voltage and positive current. That means, Q can be IGBT, BJT or MOSFET. D is obviously is diode.

Q must block Vd+Vo

D must block -Vo

To recap:

L1=L2=1.38mH

C1=172μF

VC1=Vd=16V

C2=860μF

VC2=Vo=12V

MOSFET must block 28V

Diode should block -12V

Because, the inductor values are the same with the previous topologies’ values, it is better to use same inductor to compare accurately.

The appropriate inductor is: CTDR4F-222K

It has DC resistance (DCR) 0.494Ω

IsatA=4.00A

Cost: Does not specified. Assumed it is $3/unit

Capacitor C1

16V, 180μF

860020373009

No ESR or ESL specified

Unit price: $0.16/unit

Capacitor C2

12V, 1000μF

672D108F012DS5D

0.06 ESR specified.

Unit price: $5.5

For MOSFET (Switch)

DMG3418L-7

VDSS=30V

ID=4 or 3.1A

ESR = 70mΩ

Price: $0.43/unit

For Diode

DFLS220L-7

Vr=20V

VF=0.42V at 2A

Price: $0.23/unit

The simulation results are:

Figure 28. Output voltage and current waveform

Figure 29. Inductor currents IL1 & IL2

Figure 30. C1 voltage waveform

As it can be observed, all the specifications are met and in the given range. However, duty ratio (D) is not 43%, but 48% due to losses due to non-idealities. Therefore, duty cycle ratio has been increased.

Comparison between topologies

- Create a table which shows all the circuit components that you have used in your design with their costs.

| Topology | Output Voltage Polarity | Switching Component | Diode | L1 | L2 | C1 | C2 | Total Price |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Buck-Boost Converter | − | DMG3418L-7 | SSB43L-E3/52T | CTDR4F-222K | CTDR4F-222K | 16SEPF1000M | × | $9.35 |
| Price |  | $0.43 | $0.6 | $3 | $3 | $2.32 | × |  |
| Ćuk Converter | − | DMG3418L-7 | SK44BL-TP | CTDR4F-222K | CTDR4F-222K | EEU-FR1V181B | TE1090-E3 | $12.4 |
| Price |  | $0.43 | $0.45 | $3 | $3 | $0.68 | $4.84 |  |
| SEPIC | + | DMG3418L-7 | DFLS220L-7 | CTDR4F-222K | CTDR4F-222K | 860020373009 | 672D108F012DS5D | $12.32 |
| Price |  | $0.43 | $0.23 | $3 | $3 | $0.16 | $5.5 |  |

- According to your analytical calculations and simulation results, list the advantages and disadvantages of each converter topology and comment on the input current waveforms.

In Buck-Boost converter:

- Sharp edges on input currents create EMI problems

- Supplies a negative voltage. (Inverting converter)

In Ćuk Converter:

- Supplies a negative voltage (Inverting converter)

- Reduced EMI & bi-directional power flow

- C1 should be large like VC1=Vd+Vo (Disadvantage)

- Double-ended: Inductors placed at both the input and output side, therefore; the current on both side never goes to zero (no sharp edge)

In SEPIC:

- Supplies a positive polarity voltage

- Possible to shutdown completely (when the switch is OFF)

- Reduced EMI compared to Buck-Boost Converter

- Single-ended: An inductor placed in primary end, but not in the output end

- Which one of these three topologies would you choose considering circuit components used in the circuit? Explain why did you choose that specific topology and why did not you choose other two topologies. What might be the case in which other two topologies are more suitable?

If EMI problems are being considered first, I will go through Ćuk converter, but If a needs to shutdown input source side completely and EMI is still being considered, it is better to go for SEPIC. However, If EMI problems are not considered and money should be the case, go for Buck-Boost converter.

MATLAB .slx files can be found in the repository.

Coprights are belonging to itself